SOP-IRRI-SHU-STRT-FUM-501.00	Fumigation

Title: Seed fumigation with phosphine

Application: Phosphine fumigation is used to control infestation of storage insects in seed requests for phytosanitary certification.

Material:

- Source of phosphine gas e.g. Phostoxin tablets

- Atmospheric chambers

- Full face gas mask

- Seeds placed in an open mouth plastic container, paper bags/envelopes and seed boxes

- Gloves

- Trained pesticide applicator uniform

Quality Assurance Check:

- Check the stock of the Phostoxin tablets in terms of expiration dates.

- Check the inlet and outlet valves of the chambers.

- Check the performance of the exhaust fan of the fumigation room.

- Check the performance of the full face gas mask.

- Check the piling of bags of seeds in the chamber.

- Check the volume of the chamber to be used.

- Check that the door of the chamber is gas-tight.

- Check the sanitation of the storage area of fumigated seeds.

Procedure:

- Contain seeds in seed boxes, jute sacks and paper bags. Do not put seeds in closed plastic containers. Pile materials inside the atmospheric type chamber without tightly covering other materials.

- Close all ducts (inlet and outlet valves) of the atmospheric chambers.

- Introduce Phostoxin tablets to chamber at 1 g/m3 in designated cans.

- Close the chamber door.

- Check all ducts and chamber door for gas leakage.

- Allow fumigation for 72 hr at normal atmospheric pressure (NAP) and normal atmospheric temperature (NAT).

- De-gas the chamber for 3 hr and collect oxidized pellets in __ mm thickness poly-ethylene plastic with a 1:2 soap and water mixture.

- Remove fumigated materials from chambers.

- Store fumigated seeds in a clean, well-ventilated room at 18oC and 60% relative humidity (RH).

- Bring collected oxidized phostoxin pellets to Safety and Health Office for proper disposal following these steps;

  - Fill up Safety and Health Office Request for Disposal form.

  - Label the container of the material for disposal with; name of chemical used e.g. oxidized phostoxin pellets, number of pellets used, date of collection, date of submission and OU’s name e.g. SHU.

Calculation:

- Given: Dosage used is 2 g/m3

- Problem: Solve for the number of pellets of phostoxin

Solution:

- Measure the volume of chamber. Volume is equal to Length x Width x Height (in meter).

- Multiply the volume obtained with the recommended dosage. One pellet produces 1 g of phosphine gas.

Safety:

- Wear complete personal protective equipment like gloves, mask, and uniform during setting up and degassing.

- Always conduct fumigation with an observer or in pairs.

- Do not inhale oxidized pellets, residues may contain considerable amount of phosphine gas.

- Always wash your hands with soap after the conduct of fumigation.

- Post warning sign on outside door of the chamber and door of the fumigation room with informations; date of fumigation, time of fumigation, date of de-gassing, date of re-entry, number of chamber, name of OU, name of fumigator(s) and emergency calling number.

Data entry:

- Fill out the warning sign board

- Fill up the date of fumigation in the SHU processing label on the container

- Fill up the date of fumigation of the fumigation record book

- Update the data entry in the computer

- Fill up the date of fumigation on the Application form

Reference(s):

Attachment(s):

- Copy of Warning Sign Sheet

- Copy of Safety and Health Office Request for Disposal

Prepared by:	Carlos Correos Huelma		Date: 1 October 2007

Approved by: 						Date:

Issued by:							Date: